ECE 526: HW 4

Out-of-Step Detection

Due session 25 (April 13)

System Data:

Primary nominal voltage

Nominal frequency				60 Hz

Voltage Transformer Ratio

Current Transformer Ratio

Primary pos.-seq. line impedance

Sending source transient impedance

Receiving source transient impedance

Voltage behind the transient impedance Z1S	3 % above nominal

Voltage behind the transient impedance Z1R	3  % below nominal

Load angle between ES and ER			40 deg

- Calculate nominal secondary voltage.

- Calculate ES secondary voltage.

- Calculate ER secondary voltage.

- Calculate the secondary line impedance.

- Calculate the voltage and current at the relay location for the 40 deg load angle.

- Calculate the apparent impedance at the relay location for the 40 deg load angle.

- Calculate the 3-phase apparent power seen by the relay at S.

- Set the Zone 2 reach at 120% of the line impedance and plot the line impedance and Mho Distance element characteristic in the impedance plane.

- In the same impedance plane, plot the swing locus of the apparent impedance and identify the apparent impedance calculated in 6).

- Calculate the inner and outer right blinder and out-of-step timer settings to identify swings slower than 4 Hz.

- Plot the inner and outer right blinders in the same impedance plot.

- Plot 2 seconds of the secondary voltages and currents at the relay location for slip frequency = 4 Hz.